# MICROMOUSE ROBOT — MASTER PROJECT PROMPT

## Hardware Specification, Firmware Rules & Development Workflow

**Document role:** This file is the authoritative project specification for the Micromouse robot firmware.

**Primary target:** ESP32-C6 (Espressif DevKitC-1), Arduino/C++ environment.

## 0. INSTRUCTIONS TO THE AI / ANTIGRAVITY AGENT

Treat this document as the **source of truth for the hardware configuration and development constraints**.

### Mandatory rules

1. Do NOT change GPIO assignments unless explicitly requested by the user.

2. Do NOT invent hardware specifications that are not provided.

3. If a required specification is missing, identify it as `TBD` and ask for it or clearly isolate the assumption.

4. Do NOT change I2C addresses defined in this document.

5. Never drive an ESP32-C6 GPIO above 3.3 V.

6. Do not drive motors directly from ESP32 GPIOs.

7. Motor power and logic power are separate; they share a common GND.

8. The three VL53L0X sensors MUST be initialized sequentially using their XSHUT pins because all sensors start at address `0x29`.

9. Keep hardware abstraction separate from navigation/maze logic.

10. Prefer modular, testable firmware over a single large `main.cpp`.

11. Before adding a library/API, verify that it is appropriate for ESP32-C6 and the Arduino framework.

12. When modifying existing code, preserve working hardware interfaces unless the user explicitly asks for a redesign.

13. When uncertain about a hardware behavior, explain the uncertainty instead of silently guessing.

14. Never silently remap pins, invert motor direction, change sensor addresses, or change voltage assumptions.

# 1. SYSTEM OVERVIEW

## 1.1 Main controller

MCU: **ESP32-C6**

Board: **Espressif ESP32-C6-DevKitC-1**

GPIO logic level: **3.3 V**

Firmware language: **C++**

Framework: **Arduino / Arduino-compatible ESP32-C6 environment**

## 1.2 Main peripherals

The robot consists of:

2 × GA12-N20 geared DC motors

2 × quadrature Hall-effect motor encoders

1 × DFRobot DRI0044 / TB6612FNG motor driver

1 × MPU-6050 IMU

3 × VL53L0X ToF distance sensors

1 × 2S Li-ion/LiPo battery

1 × 5 V buck converter (MP1584EN / LM2596)

1 × ESP32-C6 DevKitC-1

## 1.3 Functional concept

The robot uses:

**Encoders** for wheel speed and odometry

**MPU-6050** for inertial/angular information

**VL53L0X sensors** for wall/distance detection

**Motor driver** for bidirectional motor PWM control

**ESP32-C6** as the central controller

A future control/navigation stack for:

- motor control

- PID velocity control

- wall following

- odometry

- maze mapping

- maze solving

- motion planning

# 2. POWER ARCHITECTURE

## 2.1 Battery

Battery:

Type: 2S Li-ion / LiPo

Nominal voltage: **7.4 V**

Fully charged voltage: **8.4 V**

## 2.2 Motor power

The 2S battery supplies the motor driver's `VM` motor-power input directly.

2S Battery (+) ───────────────> DRI0044 VM

2S Battery (-) ───────────────> Common GND

## 2.3 ESP32 power

The 2S battery must NOT be connected directly to the ESP32 5 V input.

Use a buck converter:

2S Battery

│

▼

Buck Converter

│

├── OUT+ = precisely 5.00 V ──> ESP32-C6 5V/VBUS input

└── OUT- ─────────────────────> Common GND

**Important:** Verify the buck converter output is approximately **5.00 V before connecting it to the ESP32.**

## 2.4 3.3 V peripherals

The ESP32-C6 3.3 V rail supplies:

DRI0044 logic VCC

Left encoder

Right encoder

MPU-6050

VL53L0X sensors

All devices share a common GND.

### Power rule

The 3.3 V rail must not be assumed to have unlimited current capacity. If the combined peripheral current approaches the board/regulator capability, use an appropriate external 3.3 V regulator.

# 3. AUTHORITATIVE GPIO PIN MAP

## 3.1 Motor driver

| Function | ESP32-C6 GPIO |

|---|---:|

| Left motor PWM | GPIO 18 |

| Left motor DIR | GPIO 19 |

| Right motor PWM | GPIO 20 |

| Right motor DIR | GPIO 21 |

Firmware constants:

constexpr uint8_t PIN_MOTOR_LEFT_PWM  = 18;

constexpr uint8_t PIN_MOTOR_LEFT_DIR  = 19;

constexpr uint8_t PIN_MOTOR_RIGHT_PWM = 20;

constexpr uint8_t PIN_MOTOR_RIGHT_DIR = 21;

## 3.2 Left encoder

| Function | GPIO |

|---|---:|

| Encoder A | GPIO 0 |

| Encoder B | GPIO 1 |

constexpr uint8_t PIN_ENC_LEFT_A = 0;

constexpr uint8_t PIN_ENC_LEFT_B = 1;

## 3.3 Right encoder

| Function | GPIO |

|---|---:|

| Encoder A | GPIO 2 |

| Encoder B | GPIO 3 |

constexpr uint8_t PIN_ENC_RIGHT_A = 2;

constexpr uint8_t PIN_ENC_RIGHT_B = 3;

Encoder signals are quadrature signals:

Channel A: pulse/timing reference

Channel B: direction/phase information

## 3.4 I2C bus

One shared I2C bus is used by:

MPU-6050

VL53L0X left

VL53L0X front

VL53L0X right

| I2C signal | GPIO |

|---|---:|

| SDA | GPIO 6 |

| SCL | GPIO 7 |

constexpr uint8_t PIN_I2C_SDA = 6;

constexpr uint8_t PIN_I2C_SCL = 7;

## 3.5 VL53L0X XSHUT

| Sensor | XSHUT GPIO | Final I2C address |

|---|---:|---:|

| Left | GPIO 10 | `0x30` |

| Front | GPIO 5 | `0x31` |

| Right | GPIO 11 | `0x32` |

constexpr uint8_t PIN_XSHUT_LEFT  = 10;

constexpr uint8_t PIN_XSHUT_FRONT = 5;

constexpr uint8_t PIN_XSHUT_RIGHT = 11;

# 4. I2C ADDRESS MAP

| Device | Address |

|---|---:|

| VL53L0X default | `0x29` |

| VL53L0X Left | `0x30` |

| VL53L0X Front | `0x31` |

| VL53L0X Right | `0x32` |

| MPU-6050 | `0x68` |

The three VL53L0X sensors cannot remain simultaneously at `0x29`.

# 5. VL53L0X INITIALIZATION SEQUENCE

This sequence is mandatory.

## Step 1 — Disable all ToF sensors

Set:

XSHUT_LEFT  = LOW

XSHUT_FRONT = LOW

XSHUT_RIGHT = LOW

This ensures all three sensors are inactive.

## Step 2 — Start I2C

Initialize:

SDA = GPIO 6

SCL = GPIO 7

## Step 3 — Initialize Left sensor

XSHUT_LEFT = HIGH

The left sensor appears at:

0x29

Initialize it and change its address:

0x29 → 0x30

## Step 4 — Initialize Front sensor

XSHUT_FRONT = HIGH

The front sensor now appears at:

0x29

Change:

0x29 → 0x31

## Step 5 — Initialize Right sensor

XSHUT_RIGHT = HIGH

The right sensor appears at:

0x29

Change:

0x29 → 0x32

## Final state

Left  = 0x30

Front = 0x31

Right = 0x32

The sensors can then operate on the same I2C bus.

# 6. BOOT / HARDWARE INITIALIZATION WORKFLOW

Recommended initialization order:

BOOT

│

├── Configure safety/default GPIO states

│

├── Disable all VL53L0X using XSHUT

│

├── Initialize I2C

│

├── Initialize VL53L0X Left

│      └── assign 0x30

│

├── Initialize VL53L0X Front

│      └── assign 0x31

│

├── Initialize VL53L0X Right

│      └── assign 0x32

│

├── Initialize MPU-6050 at 0x68

│

├── Initialize motor driver GPIO

│

├── Initialize PWM

│

├── Initialize encoder GPIO

│

├── Attach encoder interrupts

│

└── Enter main control loop

# 7. FIRMWARE ARCHITECTURE

The firmware should be modular.

Recommended structure:

src/

│

├── main.cpp

│

├── config/

│   └── pins.h

│

├── hardware/

│   ├── motor_driver.*

│   ├── encoder.*

│   ├── imu.*

│   └── tof.*

│

├── control/

│   ├── pid.*

│   ├── velocity_controller.*

│   └── wall_controller.*

│

├── navigation/

│   ├── odometry.*

│   ├── maze_map.*

│   ├── maze_solver.*

│   └── motion_planner.*

│

└── utils/

├── timing.*

└── filters.*

The exact directory structure may be adapted to the project, but the separation of responsibilities should remain.

# 8. SOFTWARE LAYERS

## Layer 1 — Hardware abstraction

Responsible for direct hardware access:

GPIO

PWM

I2C

encoder interrupts

motor driver

IMU

ToF sensors

## Layer 2 — Sensor processing

Responsible for:

encoder pulse decoding

wheel speed calculation

distance filtering

IMU processing

sensor validity checks

## Layer 3 — Control

Responsible for:

motor PWM control

wheel velocity PID

differential-drive control

wall-following control

## Layer 4 — State estimation

Responsible for:

wheel odometry

heading estimation

robot position

sensor fusion where appropriate

## Layer 5 — Navigation

Responsible for:

detecting walls

maintaining the maze map

selecting movement direction

maze solving

## Layer 6 — Motion planning

Responsible for:

acceleration/deceleration

cell-to-cell movement

turns

stopping

velocity profiles

# 9. DEVELOPMENT WORKFLOW

When implementing a new feature, follow this workflow.

### Step 1 — Understand the hardware

Read this document first.

Do not start coding until the required hardware interface is understood.

### Step 2 — Check dependencies

Identify:

required library

required GPIO

I2C address

timing requirements

interrupt requirements

voltage requirements

### Step 3 — Implement hardware driver

Create a small isolated driver.

Example:

VL53L0X driver

↓

test initialization

↓

test distance reading

### Step 4 — Test hardware independently

Before integrating with navigation:

test motors

test encoders

test IMU

test ToF sensors

### Step 5 — Add control logic

Example:

Encoder

↓

Wheel speed

↓

PID

↓

PWM

↓

Motor

### Step 6 — Integrate sensors

Example:

VL53L0X + Encoder + IMU

↓

Robot state

### Step 7 — Add navigation

Only after the lower-level hardware and control systems are stable.

# 10. TESTING PRIORITY

Test in this order:

1. Power

2. ESP32 boot

3. I2C bus

4. VL53L0X initialization/address assignment

5. MPU-6050

6. Motor driver

7. Encoder signals

8. Motor + encoder closed loop

9. PID

10. Wall sensing

11. Wall following

12. Odometry

13. Maze mapping

14. Maze solving

15. Full motion planner

Do not debug high-level maze logic while low-level hardware is known to be unreliable.

# 11. SAFETY / FAILURE HANDLING

Firmware should fail safely.

At boot:

Motors should remain stopped until initialization succeeds.

Invalid sensor initialization should be reported.

I2C failures should not silently produce valid-looking measurements.

Encoder values should be checked for unreasonable values.

Sensor readings should have validity handling.

Motor commands should have configurable limits.

A watchdog or equivalent recovery mechanism may be added where appropriate.

If a critical hardware component fails, the robot should prefer stopping the motors over continuing uncontrolled motion.

# 12. DIAGNOSTIC MODE

A dedicated diagnostic/test mode is recommended.

It should eventually allow testing:

[1] I2C scan

[2] VL53L0X test

[3] MPU-6050 test

[4] Left motor test

[5] Right motor test

[6] Encoder test

[7] PWM test

[8] PID test

[9] Sensor telemetry

Diagnostic code should be kept separate from the normal autonomous navigation state machine.

# 13. CURRENTLY UNDEFINED / TBD PARAMETERS

Do NOT invent these values.

They must be supplied before accurate motion-control calculations are finalized.

## Motors

Motor rated voltage: `TBD`

Gear ratio: `TBD`

Motor maximum RPM: `TBD`

Motor direction convention: `TBD`

## Encoders

Encoder CPR/PPR: `TBD`

Counts per revolution after quadrature decoding: `TBD`

Encoder position relative to gearbox: `TBD`

## Robot geometry

Wheel diameter: `TBD`

Wheel circumference: `TBD`

Wheel-to-wheel distance / track width: `TBD`

Robot width: `TBD`

Robot length: `TBD`

Maze cell size: `TBD`

## Control

Maximum motor PWM: `TBD`

Target wheel velocity: `TBD`

PID Kp: `TBD`

PID Ki: `TBD`

PID Kd: `TBD`

Control-loop frequency: `TBD`

Do not hard-code these values until they are confirmed.

# 14. CODING CONVENTIONS

Prefer:

`constexpr` for fixed hardware configuration

strongly named constants

small functions

clear module boundaries

non-blocking timing where possible

deterministic control loops

explicit error handling

comments explaining hardware-specific behavior

Avoid:

unexplained magic numbers

excessive global mutable state

long blocking `delay()` calls in control loops

mixing maze algorithms with GPIO code

silently recovering from hardware errors without reporting them

# 15. CHANGE MANAGEMENT

When proposing a hardware or firmware change:

1. State what is being changed.

2. Explain why.

3. Identify affected modules.

4. Identify any GPIO/I2C/power implications.

5. Preserve backward compatibility where practical.

6. Do not modify the authoritative hardware map without explicit confirmation.

For potentially destructive changes, stop and ask before applying them.

# 16. COMMUNICATION STYLE FOR THE AGENT

When helping with this project:

Be technically precise.

Distinguish confirmed hardware facts from assumptions.

Refer to the GPIO/address tables in this document rather than inventing pin assignments.

When debugging, start from the lowest hardware layer and move upward.

Explain the reason behind a change, not just the code.

Prefer incremental changes that can be tested independently.

If a requested feature conflicts with this specification, explicitly point out the conflict.

If the hardware specification is incomplete, use `TBD` rather than fabricating a value.

# 17. QUICK REFERENCE

MCU:

ESP32-C6 DevKitC-1

LOGIC:

3.3 V

BATTERY:

2S Li-ion/LiPo

7.4 V nominal

8.4 V full

ESP32 POWER:

Buck converter → 5.00 V → ESP32 5V input

MOTOR DRIVER:

DRI0044 / TB6612FNG

MOTOR GPIO:

Left  PWM = GPIO18

Left  DIR = GPIO19

Right PWM = GPIO20

Right DIR = GPIO21

ENCODERS:

Left  A/B = GPIO0 / GPIO1

Right A/B = GPIO2 / GPIO3

I2C:

SDA = GPIO6

SCL = GPIO7

VL53L0X XSHUT:

Left  = GPIO10 → 0x30

Front = GPIO5 → 0x31

Right = GPIO11 → 0x32

MPU-6050:

0x68

VL53L0X DEFAULT:

0x29

COMMON GND:

Yes

PRIMARY DESIGN PRINCIPLE:

Hardware → Sensor Processing → Control → State Estimation → Navigation → Motion Planning

# 18. END OF MASTER SPECIFICATION

This document is the current authoritative reference for the Micromouse hardware and firmware workflow.

When the user provides new confirmed hardware information, update the relevant `TBD` fields and preserve the existing verified GPIO and address assignments unless explicitly changed.